# Chiwire Maps 5: Maintenance

For whoever keeps the device working: how the project is organised, what to update and when, and how to find problems.

## Contents

- Project structure

- Updating the map app on the card

- Updating the firmware and its libraries

- Updating the magnetic model

- Refreshing the map tiles

- Refreshing the search data

- Running the tests

- Debugging

- Known limits and open work

## Project structure

| Path | What it is |
| --- | --- |
| firmware/src/main.cpp | Start-up, Wi-Fi network, GPS reading, screen, button, LED |
| firmware/include/routes.h | The web server's addresses: the map, /location, /files, /download, /upload, /storage, the tiles, the static files |
| firmware/include/index.h, files.h | The map page and Files page shells (a change needs a reflash) |
| firmware/include/pathutil.h | Pure helpers for paths, file names and sizes, tested on the PC |
| firmware/include/Middlewares.h, MiddlewareSystem.h | Request logging and the low-memory guard |
| firmware/include/StatusLed.h | The RGB LED driver |
| firmware/test/ | Unit tests for pathutil.h (run on the PC) |
| sdCard/nav.js | The map app: layers, grid, panel, go-to, marks, search panels |
| sdCard/geo.js | UTM, grid convergence, magnetic declination, sun times, coordinate parsing |
| sdCard/marks.js, search.js | The marks/routes data model, and the search logic |
| sdCard/wmm.js | The World Magnetic Model coefficients |
| tileDownloader/spain_tiles.py | The IGN tile downloader, with its plan and area files |
| tools/build_search.py | Builds the search data from OpenStreetMap |
| tools/make_wmm_js.py, make_icon.py, make_guides.sh, card_copy_progress.py | Small helpers: the magnetic model, the icon, the guides, the copy progress |
| tests/ | Web, browser end-to-end, downloader and search builder tests |
| docs/ | The guides and the work plan |

The design rule: the dongle only stores and sends files. The maths, the drawing and the search run in the phone's browser.

## Updating the map app on the card

The map app (sdCard/*.js) lives on the card, so most changes need no reflash. After any change, copy all the app files, not only the changed ones:

cp sdCard/{*.js,favicon.ico,icon-192.png} /mnt/beacon/ && sync

A stale file on the card (for example an old nav.js next to a new search.js) can hide features without any error. The phone re-checks the app files on every visit (Cache-Control: no-cache), so a reload picks up the new version.

## Updating the firmware and its libraries

The firmware uses the Arduino-ESP32 core 2.0.x and these libraries (firmware/platformio.ini):

| Library | Notes before updating |
| --- | --- |
| TFT_eSPI | The screen settings are in build_flags (USER_SETUP_LOADED). USE_HSPI_PORT is required on the ESP32-S3; without it the dongle crashes at start-up |
| ESPAsyncWebServer and AsyncTCP (esp32async) | The original me-no-dev versions were removed from the registry. The newer API returns const headers and parameters, which Middlewares.h already uses |
| TinyGPSPlus, ArduinoJson, QRCode | Stable |

Rules:

- Update one library at a time, then build, flash and test on the device.

- Prefer versions that still support core 2.x. Moving to core 3.x is a bigger change; test it on its own.

- exFAT stays unsupported in this core build (FF_FS_EXFAT 0): the card must stay FAT32.

- After pio run, check the memory use (about RAM 14.0%, Flash 29.9% today). The dongle has no PSRAM; free heap while running is about 160–170 KB.

## Updating the magnetic model

The World Magnetic Model 2025 is valid from 2025.0 to 2030.0. After that, the north box shows "EXPIRED", and the declination slowly becomes wrong.

When NOAA and BGS release WMM2030 (expected in late 2029):

- Download the coefficient file (WMM.COF) from the NOAA World Magnetic Model page.

- Convert it: python3 tools/make_wmm_js.py path/to/WMM.COF sdCard/wmm.js

- Update the test values: replace tests/web/fixtures/WMM2025_TestValues.txt with the new model's official test values, and adjust the test that reads them.

- Run the web tests (see below); they must pass against the official values.

- Copy sdCard/wmm.js to the card.

## Refreshing the map tiles

IGN updates the MTN, PNOA and Base maps from time to time. The downloader skips tiles already on disk, so to refresh an area:

- Delete that area's tiles from sdCard/tiles/<layer>/ (or download into a new folder).

- Run the download again with the same command as in guide 1; it fetches only what's missing.

- Copy to the card with the same rsync command; it copies only files that changed.

What to expect from IGN, measured during the full download:

- No published speed limit. 24 tiles per second ran clean. The adaptive mode halves the speed when more than 1% of requests in a 2-minute window fail, then climbs back to 90% of it. It stops with exit code 3 if most requests fail even at the lowest speed: that could be a block, so wait before trying again.

- 502 Bad Gateway for single tiles, at any speed, about 0.01% of tiles. The same tile usually works minutes or hours later.

- 403 AccessDenied from the vector server for tiles that don't exist (open sea). The downloader counts them as missing, not failed.

- Our own network errors (Wi-Fi drops, waking from sleep) pause the download without changing the speed.

- Every window is logged as one row in download_metrics.csv in the output folder. Failed tiles are listed in failed.txt; python3 spain_tiles.py retry fetches them again.

Never use proxies or other tricks to go around IGN's limits.

## Refreshing the search data

OpenStreetMap changes every day. To refresh the search (a few times a year is plenty):

- Download the latest spain-latest.osm.pbf from Geofabrik.

- Rebuild: tools/.venv/bin/python tools/build_search.py tools/data/spain-latest.osm.pbf sdCard/search (about 5 minutes; needs about 7.4 GB of RAM).

- Replace the card's search/ folder: rsync -rt --modify-window=1 --delete sdCard/search/ /mnt/beacon/search/

The search logic is in two places that must agree: tools/build_search.py (on the PC) and sdCard/search.js (on the phone). The fixture tests/search/fixtures/normalize_cases.json is checked by both test suites.

## Running the tests

From the repository root:

| Suite | Command | Needs |
| --- | --- | --- |
| Web app | node --test "tests/web/*.test.js" | Node.js |
| Browser end-to-end | cd tests && node --test "e2e/*.test.js" | Node.js, Chrome or Chromium, Python 3 |
| Downloader | python3 -m unittest discover -s tests/downloader | Python 3 |
| Search builder | tools/.venv/bin/python -m unittest discover -s tests/search | the tools/.venv with pyosmium |
| Firmware helpers | cd firmware && ~/.platformio/penv/bin/pio test -e native | g++ |
| Firmware build | cd firmware && ~/.platformio/penv/bin/pio run | PlatformIO |

The maths is tested against official reference values: the NOAA/USNO sun tables, GeographicLib for grid convergence, and the WMM test values. The end-to-end tests run the real map page in headless Chrome against tests/e2e/fake_dongle.py, a small server that behaves like the firmware's routes.h. When you change routes.h, change the fake too.

## Debugging

The serial log. With the dongle on USB, ~/.platformio/penv/bin/pio device monitor shows:

- SD card initialized. or Card Mount Failed at start-up;

- SD card: … bytes free of … (measured in N ms);

- AP IP address: 192.168.4.1;

- every request, and GPS: N chars/s, N sentences ok, N failed checksum every 10 seconds. The chars/s should match what the GPS sends (about 5000 for this module), and failed checksums should stay near 0.

Opening the monitor can reset the dongle on some systems. To read the log without a reset, start the monitor first, then plug in the dongle.

Crashes. The monitor decodes crash backtraces on its own (monitor_filters = esp32_exception_decoder). For a backtrace copied from elsewhere, use addr2line from the ESP32 toolchain with firmware/.pio/build/ESP32-S3-DevKitC-1/firmware.elf.

Memory. http://192.168.4.1/diagnostics returns the free heap and the uptime. Below 40 KB free, every request gets "Service Unavailable: Low Memory" (503) until memory recovers; the threshold is criticalHeapThreshold in Middlewares.h.

GPS. http://192.168.4.1/location returns the raw GPS state: fix, latitude, longitude, altitude, satellites, hdop, speed_kmh, course. fix is true only for a position less than 5 seconds old.

## Known limits and open work

The device limits are listed in guide 1. The open device tests, measurements and ideas (marks saved on the card, search improvements, a Meshtastic mesh link) are in TODO.md.